chapter 1  Energy consumption in South Asia

Aerosols and sulphur dioxide (SO2) play an important role in regional and global climate change, with some aerosol constituents resulting in cooling and others in warming of Earth’s atmosphere. In contrast to greenhouse gases, aerosols have short atmospheric lifetimes (about few days to a week), and would concentrate in source regions, having climatic effects with strong spatial and temporal variations. Aerosol emissions inventory with good spatial resolution is a first step to understand their transport and climatic effects, on a regional scale (Chandra et al, 2001).

Aerosol and Sulphur dioxide emissions affect the earth’s atmosphere and their emission inventory is a first step towards better understanding their transport and climatic effects.

As part of preparation of SO2 and black carbon (BC) inventory, a fuel consumption database was constructed for South Asia. This includes coal, lignite, petroleum fuels and natural gas and bio-fuel consumption in different sectors.

The South Asian region, which comprises Bangladesh, Bhutan, India, Maldives, Nepal, Pakistan, and Sri Lanka, is currently experiencing a rapid growth in energy demand, concomitant with economic growth and industrialization.

The type of fuel consumed amongst the countries of the region varies significantly. Coal constitutes as much as 30% of the total commercial energy requirement in India. In Bangladesh, oil (53%) and natural gas (47%) are the dominant fuels (the shares refer to the total energy consumption). The pattern is similar in Pakistan; oil constitutes 46% and natural gas 38% of the total energy consumption. Sri Lanka is largely dependent on oil (87%), whereas Nepal uses both oil, which is largely imported, and hydroelectricity. A significant change in the fuel structure in recent years has been the increase in the share of natural gas, because of the important gas discoveries made in India, Pakistan, and Bangladesh and the environmental considerations arising from the large quantities of coal consumed in the region. At present, gas accounts for only about eight per cent of the total energy consumption in India whereas it is the main source of energy in Bangladesh (Amit K. 2000).

Significant percentage of energy demand for this region is also satisfied by biofuels. For example, bifuels, including wood, crop waste and dungcake account for 47% of the total energy consumption in India (TEDDY, 1997) and are the major source (85–90%) of cooking-energy in rural India (TEDDY, 1997).

Fuel consumption database was constructed for India at state level and for other South Asian countries at national level. This chapter describes the method adopted for database construction and consumption statistics in the South Asian region.

Energy consumption in India

In India, the country’s total primary energy supply (TPES) over the decades has kept pace with the rising income and population. It has grown from a modest figure of 147.5 million tonnes of oil equivalent (MTOE) in 1970 to approximately 437.69 MTOE in 2000, thereby growing at an average annual growth rate of 5.19% (TEDDY, 2002/03). The country has seen an expansion in the total energy use for the past five decades, with a shift from non-commercial to commercial energy sources. The share of commercial energy has risen over the years from nearly 41.02% to 68.23% of TPES in spite of dominance of non-commercial forms of energy in rural India (TEDDY, 2002/03).

Coal is the most abundant among all the commercial energy sources, and the bulk of the energy needs of the rural domestic sector are met through non-commercial fuels such as fuelwood and dung.

Fossil fuel consumption

Coal has taken center-stage in India’s energy scenario. Its dominance continues unabated primarily because power generation favours this fuel, given its indigenous availability.

Coal is the most abundant among all the commercial energy sources, and its production has increased at an average annual growth rate of 5.11%, during 1970-2001. Its share in the total commercial energy supply has, however, declined from a level of 60.47% (36.48 MTOE) in 1970-71 to 44.8% (133.89 MTOE) in 2001/02 primarily due to increase in the share of inferior grade coal in the overall coal production (TEDDY, 2002/03).

Indigenous coal has always been the most stable and least expensive option for meeting bulk of the India’s energy needs, especially for power generation. About 70% of coal produced is consumed by the power sector. In addition, other industries like steel; cement, fertilizer, chemicals and thousands of medium and small-scale industries also depend on coal for their process and energy requirements. The coal consumption has increased from 32 MMT in 1950 to 313 MMT in 2000-01, which shown in Figure 1.1.

Figure 1.1 Coal Consumption in India

In the transport sector, though direct consumption of coal is reducing due to phasing out of steam locomotives, the energy requirement for electric traction is still dependent on coal (MoC, 2003).

Despite the discovery of new sources of unconventional energy, petroleum remains one of the primary energy sources in India. The petroleum and natural gas sector has registered a robust growth in domestic production and supply over the ten years, though the country continues to depend on crude oil imports. Domestic oil production has registered a growth of nearly 5.33% per annum during the 1970-2000 period, though India’s net import of crude oil has increased from 12.43 million tonnes to 75 million tonnes in the same period (MoPNG, 2002). This is because the consumption of petroleum products has increased in all sectors, especially in the transport and industry sector.

The petro-products consumed and their major end use is shown in Table 1.1.

Table 1.1 Major end use of petroleum products

| Product | Major end use |
| --- | --- |
| LPG | Domestic Fuel. Also for Industrial application where technically essential. Now permitted as auto fuel |
| MS | Fuel for passenger cars, taxies, two & three wheelers. |
| ATF | Fuel for aircrafts. |
| SKO | Fuel for cooking & lighting. |
| HSD | Fuel for transport sector (Railways/Road), agriculture (tractors, pumpsets, threshers, etc.) and Captive power generation. |
| LDO | Fuel for agricultural pumpsets, small industrial units, start up fuel for power generation. |
| FO/LSHS | Secondary fuel for Thermal Power Plants, Fuel/ feedstock for fertiliser plants, industrial units. |

source    PPAC

Bio-fuel consumption

The bulk of the energy needs of the domestic sector are met through non-commercial fuels such as fuelwood and dung. Rural household are dependent on biomass fuels such as fuelwood, cow dung and agricultural residue. The rural household sector, which is the largest energy- consuming sector accounting for 75% of the total energy consumed by the domestic sector, depends largely on bio-fuels (TEDDY, 2002/03).

Sectoral consumption

The major sectors that consume energy includes residential, industrial, agricultural, and transportation, and these sectors continues to drive India's energy usage upwards at a faster rate.

Domestic sector

In Indian households, energy is mainly used for cooking, heating water and space, lighting, and preparing cattle feed. Cooking and lighting are the two main functions that consume energy, while importance of the other functions varies by region.

In urban areas, households use mainly three sources, namely, firewood, kerosene, and LPG as primary sources of energy for cooking. LPG is found to be the main source with 44% of households using it as a primary source for cooking. About 22% of households use firewood and chips as primary source of energy for cooking. The per capita monthly consumption of firewood and chips in urban areas has increased from 6.09 kg in 1993/94 to 6.45 kg in 1999/2000 (NSSO, 1997 & 2000). This trend shows that urban poor households are still dependent on traditional fuels for their energy needs.

In rural areas, according to the latest NSSO survey (1999-2000), 86.1 % of the rural households use fuelwood and dung for cooking. The comparison of the last two NSSO rounds (1993-94 & 1999-00) reveals that there is a slight decrease in the consumption of biomass-based fuels for cooking. However, the decrease is negligible compared to share of biomass-based fuels. There is a slight penetration of LPG in rural areas with an increase of 3.5% in the number of households using LPG for cooking.

Power sector

The Indian power sector, the backbone of nation’s development, has gradually evolved over the years. After independence, India’s installed generation capacity was mere 1.3 GW (gigawatt), while today it is more than 100 GW. Annual gross generation in utilities has grown at an average annual growth rate of approximately 7.7%, during 1975-2001. Over the years, thermal power has continued its dominance over the aggregate power generation, and its share has increased from 54% to nearly 81% during the same time period (TEDDY, 2002/03). Industry is the largest consumer of power and its consumption has grown at the rate of nearly 4.2 % per annum in the last quarter of the 20th century.

Primary fuel that is consumed in power sector in India is coal; power sector consumes approximately seventy percent of coal. Apart from coal the other fuels that are consumed in the power sector are lignite, diesel oil, furnace oil, heavy stock, naphtha and natural gas.

Industry sector

The industrial sector consumes about 50% (including feed stocks) of the total commercial energy produced in the country. The total industrial energy consumption, including non-energy uses, grew from 45.7 MTOE (million tonnes of oil equivalent) in 1984/85 to 113.1 MTOE in 1996/97. Of the commercial sources of energy, coal and lignite contribute about 57%, oil and natural gas 33% (TEDDY 2002/03).

The energy intensive industries are iron & steel, cement, fertilizer, chemical & allied industries. The fuels that are consumed in these industries are coal, diesel oil, furnace oil & heavy stock.

Agriculture

Increased cropping intensity and modern agriculture necessitated higher energy inputs for field operations as well as agro-processing. One of the major engineering inputs adopted has been irrigation pump. The number of electrical and diesel pump need has increased from 1.6 million in 1970 to 13.0 million in 2001/02 (TEDDY, 2002/03).

Animal power has been supplemented by tractors, power tillers, diesel engines, and electric motors to a considerable extent. The share of agriculture sector in the total commercial energy consumption in India has increased from 1.7% in 1953/54 to 9.0% in 1990/91. The share of commercial energy uses like electricity, diesel, fertilizer and tractor power in the total energy consumption by the agriculture sector has increased from meagre 11% in 1951 to more than 90% in 2001/02 (TEDDY 2002/03). The primary fuel consumed in this sector is diesel oil.

Transport sector

The transport mix in India – Road, Rail, Shipping and Aviation consume products of petroleum, road being the major mode of transport. There has been a tremendous increase in not only the number of registered vehicles but also the road network. In 2000, the number of registered vehicles was over 48 million (MoRTH, 2002). The transport sector consumes motor spirit (MS), diesel oil, furnace oil, and aviation fuel (ATF)

Energy consumption in other South Asian countries

Economic and population growth in South Asia have resulted in rapid increases in energy consumption in recent years.

Bangladesh has small reserves of oil and coal, but potentially very large natural gas resources. Commercial energy consumption is around 66% natural gas, with the remainder mostly oil (plus limited amounts of hydropower and coal). Only around 20% of the population (25% in urban areas and 10% in rural areas) has access to electricity, and per capita commercial energy consumption is among the lowest in the world. Non-commercial energy sources, such as wood, animal wastes, and crop residues, are estimated to account for over half of the country's energy consumption (EIA, 2004).

Pakistan's energy consumption has nearly tripled in the last 20 years. From 0.6 quadrillion Btu in 1980, Pakistan's total energy consumption in 1998 is around 1.7 quadrillion (Irfanullah, 2001). Pakistan's energy consumption by sector shows that nearly half (49%) is attributed to residential. The industrial sector consumed 33.5%, while transportation sector consumed 13.2%. Commercial energy consumption by fuel type showed that oil made up 43.5%, natural gas 38.3%, and coal just 5.1%.

Sri Lanka and the Maldives are overwhelmingly dependent on petroleum (82% and 100%, respectively. The Himalayan countries of Bhutan and Nepal have the highest shares of hydroelectric power in their energy consumption mix at 80% and 31%, respectively, in 2002 (EIA, 2004).

Data collection methodology

Fuel consumption data was collected for India at state level and for Bangladesh, Pakistan, Nepal and Sri Lanka at national level.

The data was not available for other South Asian countries – Bhutan and Maldives and therefore they were not included in the database.

In the first phase (Phase I) of this study, fuel consumption data was compiled for the period of ten years between 1990–91 and 1999-00. In the second phase (Phase II) the time period was further expanded and a time series fuel consumption data was collected starting from the year 1951 at an interval of five years till 2001. Projections for future years 2005, 2010, 2015 and 2020 were made. Methodology adopted for India and other South Asian countries is described in this section.

The fuel consumption data for India at state level and for other South Asian countries at national level, along with data sources is given in Annexure I of the report.

Methodology for India

At present India is divided into 28 states, which are administered through state governments and 7 union territories, directly administered by the union government. For the present study the division followed is 25 states and 7 union territories, the states formed in late nineties were assumed to be undivided - Chattisgarh included in Madhya Pradesh, Jharkhand in Bihar and Uttaranchal in Uttar Pradesh.

The fuel consumption in each state varies considerably depending on industrial development, urbanisation and population. Fossil fuel consumption was divided into four major sectors namely power, industrial, domestic, and transport.

Bio-fuels consumption in domestic sector was included in the database, whereas bio-fuel consumption in industries and other sectors is not included as the information/data availability is limited.

Coal and lignite

Phase I (1990-91 to 1999-2000)

The statewise consumption of coal in different industrial sectors was collected from sales & marketing report of Coal India Limited (CIL). Coal despatches to different sectors from Singareni Collieries Company Limited (SCCL) were also collected from Coal Directory of India, Ministry of Coal. The combined despatch of CIL & SCCL is approximately 91% of the total coal despatch. The rest of the despatch (approximately 9%) is either from smaller coal companies or imported coal. Only the combined despatch of CIL and SCCL was collected and included in the database for emission estimation, as the information on other sources of despatch, such as despatch destiny & end use sector is not available. The following assumptions were also made,

- Singareni Collieries Company Limited (SCCL) despatches coal to most of the customers situated in Andhra Pradesh and to some extent in other parts of Southern India. The customers are broadly categorized as core sector and non-core sector (SCCL, 2004). Since the exact despatch destination details were not available, the entire despatch was allocated to Andhra Pradesh. 80% of SCCL coal is being fed to Power Houses and about 12% goes to Cement Sector (SCCL, 2004). The rest goes to non-core sectors. Accordingly despatch was allocated to these sectors.

- It is also assumed that whatever coal is despatched from these companies is consumed.

Lignite is consumed mainly in Tamil Nadu and Gujarat. Lignite consumption from these states was collected from Coal Directory of India, Ministry of Coal. It was assumed that in Tamil Nadu lignite is consumed in power sector. In the case of Gujarat power sector consumption data was collected from CEA (Central Electricity Authority), the difference in total consumption and power sector was allocated to industries.

Phase II (1951-2020)

The total coal and lignite consumption at national level was compiled for the years 1953-54 to 1979-80 from Report of the fuel policy committee and Report of the working group on energy policy, Planning Commission. The national level coal despatch data for the year 1985-86 was compiled from Coal Directory. These national aggregates were broken to state level and apportioned to different sectors based on the ratios of 1990-91 data. CAGR between 1980-81 and 2000-01 was calculated and consumption for next 20 years, i.e. till 2020, was projected based on this CAGR.

Petro-products

Phase I (1990-91 to 1999-2000)

Indian petroleum and natural gas statistics gives information on state level consumption of different petroleum products. It also gives estimates on sector-wise end use at the national level.  The state level consumption of petro-products (used as fuel i.e. combusted - listed in Table 1.1) was extrapolated to different sectors based on national sectoral end use. The following assumptions/estimates were made,

- It is assumed that kerosene oil is consumed only in the domestic sector, and, motor spirit and aviation turbine fuel in the transport sector.

- There is no system of capturing sector-wise sale of HSD through retail outlets (Resellers). This consumption was therefore allocated to other transport (including agriculture retail trade), which is approximately 73% of total HSD consumption. However, directional figures are available based on all India study conducted by IMRB (Indian Market Research Bureau) in 1998. These figures were used by the Ministry of Petroleum and Natural Gas (MoPNG) to revise their national sectoral end use of HSD from the year 1997 –98. Due to this revision the sectoral consumption of road transport has increased from meagre 8% to almost 53% from 1997-98 onwards. The directional figures by IMRB were used to reallocate HSD consumption into different sectors for the entire study period (1990-91 to 1999-00).

- The percentage consumption of HSD, FO, LDO and LSHS/HSS in different industries – steel, cement, fertilizer and non-core at state level was estimated from Annual Survey of Industries (ASI) statistics. These ratios were used to apportion total state level industrial consumption of HSD, FO, LDO and LSHS/HSS (from MoPNG statistics) to different industries – steel, cement, fertilizer and non-core.

Phase II (1951-2020)

The total consumption of all petro-products at national level was compiled for the years 1950 to 1969 from Indian Petroleum and Petrochemical Statistics, 1969. Since product-wise, state level sectoral consumption was not available for this time period these were estimated by adopting following methodology.

- The total petro-product consumption was divided into different products based on the product to total consumption ratios of the year 1969. These products were then apportioned to different states based on the ratios of state to national consumption for the year 1975.

- The state level sectoral consumption of these products was estimated based the national average sectoral consumption for the year 1975.

For the years 1975, 1980-81 and 1985-86 state level consumption of different petroleum products was collected from Indian petroleum and natural gas statistics. The estimate on sector-wise end use at the national level for the year 1990-91 was used to apportion state level consumption into different sectors for these years.  CAGR between 1980-81 and 2000-01 was calculated and consumption for next 20 years, i.e. till 2020, was projected based on this CAGR.

Fuelwood, dung cake and crop residue

Firewood, agricultural crop residue and animal dung are commonly referred to as non-commercial forms of energy primarily used in domestic sector. It is well recognized that substantial quantities of non-commercial fuels are used in a few major industries such as sugar and in unregistered factories and household industries. It has not been possible to estimate on a reliable basis the total contribution of non-commercial fuels to energy consumption in the industries sector. The analysis of consumption of non-commercial fuels in this in this report relates entirely to their use in the household sector. Even in respect of this sector, there is no record of production or usage of such fuels and it is necessary to estimate consumption from sample surveys (Planning commission, 1974). Various alternative estimates/estimation methods of consumption of non-commercial fuels were considered and it was decided to adopt

- Estimates of the Report of the fuel policy committee, 1974, for the years.

- Estimates of the report ‘Potential for use of renewable sources of energy in Asia and their cost effectiveness in air pollution abatement’, TERI, 1999, for the years…

- Estimation based on NSSO data.

Phase I (1990-91 to 1999-2000)

National Sample Survey Organisation (NSSO) collects information on state level consumption of domestic fuels in India. The consumption of firewood & chips for the study period was estimated based on two NSSO surveys - Monthly per capita consumption of firewood & chips in urban & rural areas for the years 1993-94 & 1999-00. The compounded annual growth rate (CAGR) was calculated based on these two surveys and was extrapolated to other years of the study period. The annual consumption was estimated by multiplying population (based on population Census-1981&1991&2001 as received from Registrar General of India) with monthly per capita consumption.

The consumption of agricultural crop residue and dung cake estimate is based on an earlier study conducted by TERI (TERI Report, 1999). Consumption estimates of agricultural residue and dung cake for the years 1990 & 2000 under business as usual scenario was taken as basis.

The compounded annual growth rate (CAGR) was estimated based on these two years data and was interpolated to other years of the study period. The aggregate consumption in the Himalayan region was apportioned to the respective states based on cattle population & cultivable land area, whereas for other regions the data was available at state level.

Phase II (1951-2020)

The total consumption of firewood, dungcake and agricultural crop residue at national level was compiled for the years 1953-54 to 1975-76 from the report of the working group on energy policy, planning commission, 1979. This national aggregate consumption was allocated to different states based on the population of the states.

For the remaining years, 1980-81 & 1985-86 the consumption of non-commercial fuels was estimated based on the methodology used in the Report of the fuel policy committee 1974. Briefly, the estimation is based on the assumption that the annual per capita consumption of energy (of all forms) in the household sector is 0.38 tonnes of coal replacement (tcr) in rural areas and 0.40 tcr in the urban areas. Using the estimates of rural and urban population for each year, the total energy consumed in the household sector is worked out. From this total energy consumption the consumption of commercial energy, i.e. Coal, Oil products and electricity, in the household sector is deducted and the total non-commercial energy is derived. The consumption of firewood agriculture waste and animal dung are calculated on the assumption that their shares in the total non-commercial energy consumption are 65 per cent, 15 per cent and 20 percent respectively.

CAGR between 1990 and 2000 was calculated and consumption for next 20 years, i.e. till 2020, was projected based on this CAGR.

Methodology for other South Asian countries

Energy Statistics of Non-OECD Countries, an IEA (International Energy Agency) database, contains basic energy statistics for over 100 Non-OECD countries and 11 regions. Data are provided for energy supply and consumption in original units (1000 metric tons, terajoules and gigawatt hours) for various types of coal, oil, gas, renewables and waste. Definitions of products and flows, explanatory notes on the individual country data and net calorific values are also included. The data is available from 1971 onwards.

The United Nations publication, World Energy Supplies- statistical paper series, provides data on production, trade and consumption of energy for different countries from 1951 onwards.

These two databases were used to compile fuel consumption data for other South Asian Countries.

Phase I (1990-91 to 1999-2000)

Fossil fuel consumption data in other South Asian countries - Bangladesh, Nepal, Pakistan and Sri Lanka was collected from Energy Statistics of Non-OECD Countries, IEA.  The IEA statistics for Bhutan and Mauritius is not available. The consumption of coal and petro products in different sectors such as power, transport, industries, agricultural and domestic sectors was collected for the period 1990 to 2000.

Fuel wood consumption data for the years 1990 to 1994 was collected for all five countries, including Bhutan, from Regional Wood Energy Development Programme (RWEDP). Consumption for rest of the years was estimated based on the projected percentage growth rate under BAU scenario.

Phase II (1951-2020)

Fossil fuel consumption from 1951 –1966 at an interval of five years was compiled from UN-database. UN-database was used for this period, as IEA statistics is available from1971 onwards. Also, this database does not give details on the end use/ sectoral use of fuels. For the rest of the years 1971-1986 fuel consumption was compiled from IEA statistics and the methodology adopted was similar to phase I. CAGR between 1981 and 2001 was calculated and consumption for next 20 years, i.e. till 2020, was projected based on this CAGR.

Fuelwood consumption from RWEDP database is available from 1980 onwards. Consumption for 1951-1976 was estimated assuming the growth rates of 1981-2001. Consumption for future years was estimated based on the projected percentage growth rate under BAU scenario. The conversion factors and growth rates used for estimation are shown in Table 1.2.

Table 1.2  Conversion factors and growth rates assumed in fuelwood 
                  consumption estimation

| Country | Conversion factor, m3 to kg | % Annual growth rate |
| --- | --- | --- |
| Bangladesh | 650.4747 | 2.09 |
| Bhutan | 666.2069 | 1.50 |
| Nepal | 650.0907 | 2.61 |
| Pakistan | 651.1333 | 3.62 |
| Sri Lanka | 668.7484 | 1.47 |

source    RWEDP